DIP QUICK STAIN KIT

# PRINCIPLE OF THE KIT

This procedure is a classic variant of the Blue Thiazinic Methylene and Eosin contrast colouration (Romanowsky). The samples tested are smear of blood or of marrow, urine sediment, expectoration, epithelial tissue’s samples.

In the parasitology often it is used to underline the Plasmodium of malaria, then it can be used to underline the Pneumocistis carinii.

PACKAGE

CODE: BS156 (3 X 250 ML)

CODE: BS156A (3 X 500 ML)

CODE: BS156B (3 X 1000ML)

Composition of the Reagents: Reagent A (Methanol), Reagent B (Eosin y in phosphate buffer), Reagent C (Thiazinic Colouring in phosphate buffer)

STORAGE

Store the kit between 5-30°C in a cool and dry place. Avoid the direct exposure at the light. Keep the bottle hermetically sealed.

Stability: 24 months from the production date. In case of precipitation the solution can be dissolve in bain-marie at 60° for some minutes, the formation of a “mirror” on the wall of the bottle shows that the product is aged and it is not recovered.

ATTENTION

Reagent A:

H370: Causes organ damage.

H331: Toxic if inhaled.

H311: Toxic if in contact with skin.

H301: Toxic if ingested.

H225: Highly flammable liquid and vapor.

P210: Keep away from sources of heat, hot surfaces, sparks, naked flames or other sources of ignition. Not smoking.

P261: Avoid breathing dust/fume/gas/mist/vapours/aerosols

P280: Wear protective gloves / protective clothing / eye / face protection.

P370+P378: In case of fire: Use carbon dioxide, foam, dry chemical to extinguish.

P403+P233: Keep container tightly closed and in a well-ventilated place.

P308 + 311 in case of exposure or possible exposure: contact a doctor or a physician.

Reagent B and C

They are not classified as dangerous in accordance with the provisions of Directive 1999/45 / EC (and subsequent amendments and adjustments) and in accordance with the provisions of Regulation (EC) 1272/2008 (CLP) (and subsequent amendments and adjustments).

PRECAUTIONS		DIAGNOSTIC KIT FOR IN VITRO USE ONLY

Do not use longer the expiration date that is specified on the box.

Do not mix products of different batches.

Follow the usual safety advice necessary at the sampling, at the manipulation and at the treatment of biological samples and of all the material that were in contact with the patient.

SPECIMEN COLLECTION AND PREPARATION

FOR SMEAR:

Swipe the sample on a glass slide to obtain a cell monolayer and allow the slide to dry to air.

Escreate: expectorate the patient in a clean, wide-mouthed container, preferably in the morning as soon as the patient wakes up.

ProcedURE

- Dip the slide directly into the Reagent A ( FIXATIVE) for  5 times and for 1 second each time. After each immersion, the excess of fixative has to drained off.

- Dip The slide  into the Reagent B (EOSINIC STAIN) for 5 times and for 1 second each time. After each immersion, the excess of reagent has to drained off.

- Finally dip the slide into the Reagent C (THIAZINIC STAIN)  for 5 times and for 1 second each time. After each immersion, the excess of reagent has to drained off.

- Shake the smear to mix the dyes

- Wash with spring water.

- Rinse the smear in distilled water or in a Waise buffer pH 7,2.

- Leave the slide to dry in vertical position (do not use heat, stove or plate).

FOR SECTIONS IN PARAFFIN:

Bring the sections to the water.

PROCEDURE

1.  Color the slide with Reagent B (EOSINIC STAIN) for 30 seconds.

2. Color the slide with Reagent C (THIAZINIC STAIN) for 30 seconds.

3. Rinse quickly in distilled water

4. Dehydrate and clarify

5. Mont the slide and observe with microscope

Note: Remind to close always the bottle after use to avoid the stain’s evaporation.

INTERPRETATION OF RESULTS

For the interpretation of the chromatic picture of the staining, reference is made to the results of the resulting classical May-Grunwald-Giemsa method:

Hematology, batteriology:

NUCLEI						Violet blue

NUCLEI OF BLOOD PARASITES AND PROTOZOA	Shining red

LYMPHOCYTE PROTOPLASM			Light blue with possible purple-red granules

NEUTROFILA GRANULOCYTES 			Violet red

EOSINOPHILS					Red brown

BASOPHILS					Blue

PLATELETS					Blue with violet central body

ERYTHROCYTES					Pale pink

MICROORGANISMS				Violet blue

SPERMATOZOA					Dark blue

Note: In case of doubts in the analysis, repeat the above procedure with another sample coming from the same source as the previous one.

METHOD LIMITS

The method limits are identified in the sample collection that is analysed on the microscope. The definitive diagnosis must be carried out on clinical data and on the anamnesis of the patient, then the result of the test must be interpreted from the physician.

REMOVAL OF WASTE MATERIAL

All systems used to collect the biological sample enter in contact with the slide where is made the colouration, the medical device and its accessories, after use, are considered like special waste and must be removed with any contact with persons or animals, in accordance with the national, regional and local laws in force.

NOTE

As for all diagnostics tests, a definitive diagnosis cannot be make on the result of a single test, but must be confirmed from other clinical parameters, considering the patient’s anamnesis.

The colouring depends from the concentration and from the ratio between the stains concentrations, it can be changed from the pH, colouring and fixing times.

PRODUCER

Dyaset s.r.l. Via A. Grandi, 3 44015 Portomaggiore (Ferrara) Tel. +390532814820 Tel. +390532320002 e-mail: dyaset@dyaset.com website. http://www.dyaset.com

SYMBOLS EXPLANATION

For in vitro use only

Batch number

REF	Reference number

To be stored between X-Y°C

Expiry date (year-month)

To refer to the technical procedure

INCLUDEPICTURE  "https://encrypted-tbn2.gstatic.com/images?q=tbn:ANd9GcQjwwtVW4uSEDAQ-_5C_MFOk9v2KqPm5_DJagC9HG-iWtWi1eN2Fw" \* MERGEFORMATINET  INCLUDEPICTURE  "https://encrypted-tbn2.gstatic.com/images?q=tbn:ANd9GcQjwwtVW4uSEDAQ-_5C_MFOk9v2KqPm5_DJagC9HG-iWtWi1eN2Fw" \* MERGEFORMATINET  INCLUDEPICTURE  "https://encrypted-tbn2.gstatic.com/images?q=tbn:ANd9GcQjwwtVW4uSEDAQ-_5C_MFOk9v2KqPm5_DJagC9HG-iWtWi1eN2Fw" \* MERGEFORMATINET  INCLUDEPICTURE  "https://encrypted-tbn2.gstatic.com/images?q=tbn:ANd9GcQjwwtVW4uSEDAQ-_5C_MFOk9v2KqPm5_DJagC9HG-iWtWi1eN2Fw" \* MERGEFORMATINET  	Biological risk

| LIST CHANGES |  |  |
| --- | --- | --- |
| REV. N. | DESCRIPTION | DATA |
| 0.0 | CREATING DATA SHEET | 18/06/2008 |
| 0.1 | REVISION PROCEDURE | 19/03/2013 |
| 0.2 | REVISION PACKAGING | 01-12-2015 |
| 0.3 | INSTRUCTIONS IN REF. TO REGULATION 1272/2008 (CLP) | 15-06-2017 |
| 0.4 | REVISION PROCEDURE | 07/02/2024 |
| 0.5 | REV. ATTENTION | 30/07/2025 |